Steady-state Lumped Parameter Analysis

- 123 Thermal via (0.3mm diameter) , 6 Layer PCB, Rjc= 0.5 Ω , Rcpb= 4.5Ω(1.6mm & 6 layer), RTIM = 1.5 Ω

https://www.myheatsinks.com/calculate/thermal-resistance-plate-fin/

- The results changes between Rth (heatsink)= 0.25 - 0.45 ℃/W

FEA Analysis (Steady State)

If you think that GanFET is a black box with 0.5 Rjc ;

- L is the thickness (m) (thickness of the package and the thermal pad);

- A is the effective contact area (m2) (Area of the thermal pad) ;

- k is the conductivity of the box (W/(m*K))

R= LA.k   (1)

k=LA.R=0.440 x 10-32.667 x 10-3 x 0.5=32.99  W/(m.K)

- For the most accurate Heat convection coefficients CFD analysis required.

3D Design Engineers are divided into 2 groups;

- MCAD Design engineer (Mechanical)

- ECAD Design Engineer (Electronic)

- These engineers share the common file that is in IDF, EMN or STEP format.

- Each format has different features. (Based on the PCB layout and holes)

- IDF is the most common.

Following weeks;

- More accurate results

- FEA Transient Analysis